# Research and Selection of a Publicly Available Household Energy Consumption Dataset

Evaluation of open-source datasets by data quality, granularity, size, and relevance to household energy consumption patterns

Final recommendation: UCI Individual Household Electric Power Consumption. It offers a strong balance of fine time resolution, long observation period, manageable download size, straightforward structure, and an explicit CC BY 4.0 license.

# 1. Objective and Evaluation Criteria

Objective. Identify and evaluate suitable publicly available household energy consumption datasets, then select one dataset for use in the project.

Evaluation criteria. The datasets were compared on data quality, temporal granularity, size and computational practicality, breadth of household coverage, measurement types, openness/licensing, and relevance to analyzing household energy consumption patterns.

# 2. Candidate Dataset Evaluation

| Dataset | Coverage & granularity | Strengths | Limitations |
| --- | --- | --- | --- |
| UCI Individual Household Electric Power Consumption | 1 household in Sceaux, France; Dec. 2006–Nov. 2010 (47 months); 1-minute sampling; 2,075,259 observations; 9 features. | Long continuous period; fine one-minute resolution; multiple electrical quantities and three sub-metering channels; simple tabular structure; compressed download about 19.7 MB; CC BY 4.0. | Represents only one household, limiting population-level generalization; approximately 1.25% of rows have missing measurement values. |
| REFIT Electrical Load Measurements (Cleaned) | 20 UK households; approximately two years; 8-second sampling; aggregate household power plus appliance-level measurements. | Multiple households support cross-household comparisons; very fine resolution; cleaned version available; aggregate and appliance-level consumption are highly relevant to usage-pattern analysis; CC BY 4.0. | Much larger than UCI (cleaned archive about 490 MB); 8-second resolution may be more detailed than necessary for an introductory analysis and increases processing requirements. |
| UK-DALE | 5 UK homes; primary appliance/whole-house data at 6-second resolution, with additional 1-second and 16 kHz data for subsets. | Rich appliance-level detail; high temporal resolution; useful for load disaggregation and appliance-behavior research; open-access research resource under CC BY 4.0. | Only five homes; more complex multi-file structure; ultra-high-resolution components can be extremely large (the complete 16 kHz set is about 4 TB), making them unnecessary for a basic household-pattern project. |

# 3. Final Selected Dataset

Selected dataset: UCI Individual Household Electric Power Consumption

Selection rationale. The UCI dataset is the best fit for this project because it provides more than two million one-minute observations across almost four years while remaining considerably easier to download, inspect, clean, and model than the second-level alternatives. Its long time span supports analysis of hourly, daily, weekly, monthly, and seasonal patterns. The electrical measurements and sub-metering variables also allow both overall-consumption analysis and limited component-level analysis.

Data-quality consideration. All calendar timestamps are present, but approximately 1.25% of rows contain missing measurement values. This is a manageable issue that can be handled explicitly during preprocessing rather than a reason to reject the dataset.

Open-source suitability. The UCI repository states that the dataset is licensed under Creative Commons Attribution 4.0 International (CC BY 4.0), permitting sharing and adaptation with appropriate attribution.

| Official source | UCI Machine Learning Repository – Individual Household Electric Power Consumption |
| --- | --- |
| DOI | 10.24432/C58K54 |

# 4. Initial Understanding of the Selected Dataset

Time resolution and coverage. Measurements were collected every minute from a single household in Sceaux, France, from December 2006 through November 2010, covering 47 months. The repository reports 2,075,259 observations and nine features.

| Feature | Meaning / unit |
| --- | --- |
| Date | Calendar date (dd/mm/yyyy). |
| Time | Time of observation (hh:mm:ss). |
| Global_active_power | Minute-averaged household active power, measured in kilowatts (kW). |
| Global_reactive_power | Minute-averaged household reactive power, measured in kilowatts (kW). |
| Voltage | Minute-averaged voltage, measured in volts (V). |
| Global_intensity | Minute-averaged household current intensity, measured in amperes (A). |
| Sub_metering_1 | Active energy in watt-hours (Wh), primarily for the kitchen area/appliances. |
| Sub_metering_2 | Active energy in watt-hours (Wh), primarily for the laundry-room appliances. |
| Sub_metering_3 | Active energy in watt-hours (Wh), primarily for the electric water heater and air conditioner. |

## Analytical relevance

The selected dataset can support analyses such as:

Identifying peak and off-peak household electricity consumption periods.

Comparing consumption by hour of day, day of week, month, and season.

Examining long-term trends and recurring consumption patterns.

Comparing total household demand with the three available sub-metered areas.

Building forecasting, anomaly-detection, clustering, or regression models after appropriate preprocessing.

# 5. Acceptance Criteria Check

| Requirement | Status |
| --- | --- |
| At least 3 potential datasets with pros/cons | Met — UCI, REFIT, and UK-DALE are evaluated above with strengths and limitations. |
| Final selected dataset and source URL | Met — UCI Individual Household Electric Power Consumption is selected, with the official UCI source and DOI documented. |
| Initial understanding of features | Met — time resolution, observation period, measurement types, units, sub-metering, missing-data issue, and analytical uses are documented. |

# 6. References

1. UCI Machine Learning Repository. “Individual Household Electric Power Consumption.” Dataset page, DOI 10.24432/C58K54. Source

2. University of Strathclyde. “REFIT: Electrical Load Measurements (Cleaned).” DOI 10.15129/9ab14b0e-19ac-4279-938f-27f643078cec. Source

3. Kelly, J., & Knottenbelt, W. (2015). “The UK-DALE dataset, domestic appliance-level electricity demand and whole-house demand from five UK homes.” Scientific Data, 2, 150007. Source